# GPU Options for the AI Lab

Qwen3_6-35B-A3B (4-bit) + Gemma-4-26B-A4B-it-MXFP4

# 1. Model Requirements

Qwen3_6-35B-A3B-4bit: ~20–22 GB estimated model memory; primary deep-coding/reasoning model in the supplied report.

Gemma-4-26B-A4B-it-MXFP4: ~15–17 GB estimated model memory; low-latency model in the supplied report.

Planning: 48 GB+ is preferable for context, KV cache and multi-user serving; 80–96 GB+ gives substantially more headroom. This is an infrastructure planning conclusion, not a GPU benchmark.

# 2. GPU Comparison

| GPU | VRAM | Bandwidth | Qwen 35B | Gemma 26B | Why use | Why not |
| --- | --- | --- | --- | --- | --- | --- |
| RTX A6000 | 48 GB | ~768 GB/s | Yes | Yes | 48 GB workstation GPU; practical for a single-node lab | Older Ampere; less capable than newer L40S/Blackwell |
| A40 | 48 GB | 696 GB/s | Yes | Yes | Data-center Ampere; ECC; NVLink; possible budget option | Older; no MIG; passive cooling |
| L40S | 48 GB | 864 GB/s | Yes; strong fit | Yes; strong fit | Modern Ada; strong inference balance; 48 GB | 48 GB ceiling; no NVLink/MIG |
| A100 80GB | 80 GB | 1.94–2.04 TB/s | Yes; excellent | Yes; excellent | 80 GB HBM2e; high bandwidth; MIG up to 7 | Older generation; higher cost |
| H100 80GB | 80 GB | 3.35 TB/s (SXM) | Yes; excellent | Yes; excellent | Very high performance; mature CUDA; MIG | Very high power/cost; likely overkill for 35B inference |
| RTX PRO 6000 Blackwell | 96 GB | — | Yes; excellent | Yes; excellent | 96 GB GDDR7 ECC; current Blackwell | Premium cost; high power |
| AMD MI300X | 192 GB | 5.3 TB/s | Yes; huge headroom | Yes; huge headroom | 192 GB HBM3; exceptional bandwidth; GenAI/HPC | 750W OAM server accelerator; ROCm/platform complexity |

# 3. Brief Assessment

RTX A6000 — Good workstation/lab option when 48 GB is sufficient.

A40 — Budget-oriented data-center option, especially when acquisition cost is attractive.

L40S — Strong candidate for the first shared-inference node: modern, 48 GB, and relatively straightforward NVIDIA deployment.

A100 80GB — Strong shared-server choice when context, concurrency or multiple models justify 80 GB.

H100 80GB — Performance-first option; difficult to justify as the first GPU when minimizing initial cost.

RTX PRO 6000 Blackwell — Modern 96 GB NVIDIA option when high memory and Blackwell are worth the premium.

MI300X — High-memory scale option; its 192 GB is far beyond the current model footprint, so platform cost should be justified by future workloads.

# 4. Recommended Procurement Strategy

Initial / cost-conscious: Evaluate L40S 48 GB first; compare A40/A6000 if their acquisition price is substantially lower.

Shared high-headroom: A100 80 GB when large context, concurrency or multiple models justify the additional memory.

High-performance: H100 80 GB or RTX PRO 6000 Blackwell 96 GB when throughput/latency or newer architecture justifies the premium.

High-memory scale: MI300X when 192 GB HBM3 and 5.3 TB/s bandwidth are valuable enough to justify ROCm and server-platform complexity.

# 5. Multi-Model Routing

Gemma → fast, latency-sensitive work: autocomplete, boilerplate and simple responses.

Qwen → complex work: debugging, refactoring, repository analysis and deeper coding.

Model Router → select the model based on workload complexity and latency requirements, reducing unnecessary use of the heavier model.

# 6. Important Qualification

The table describes hardware capacity and deployment considerations, not guaranteed Qwen/Gemma tokens/sec. Actual throughput depends on quantization, context length, batching, concurrency and CUDA/ROCm/vLLM configuration. Benchmark shortlisted GPUs with representative organizational workloads before procurement.

Sources: NVIDIA and AMD official specifications; supplied AI Model & Infrastructure Evaluation Report for model memory and workload assumptions.